Field Navigation App

Developer Brief — v1

# 1. Design Intent

This app is a field tool, not a GIS application. It is for people working in caves, gorges, water systems, and other remote environments — often one-handed, in poor light, with cold or wet fingers, and with no internet.

The reference point for what this app should NOT become is Map Plus (Miocool Inc.). Map Plus is genuinely powerful but feels clunky in the field because every feature is equally available all the time. Layers, folders, scripts, attribute editing, conditional styling and dozens of file formats all compete for the user's attention. The user has to decide what mode they are in before they can do anything.

This app should feel like the opposite of that. Three principles drive every design decision:

- Speed over flexibility. If a choice exists between making the app capable of more things and making the most common action faster, choose speed.

- Calm over busy. The map fills the screen. Controls are minimal and at thumb height. Nothing flashes, animates, or demands attention unless it must.

- One-handed and foolproof. Every primary action must be doable with one thumb. Destructive actions must be impossible to trigger by accident.

If a feature does not directly serve a person standing in a cave or gorge trying to capture spatial data quickly, it does not belong in v1.

# 2. Platform and Stack

- Platform: iOS, iPhone-first. iPad support is acceptable but not optimised in v1.

- Minimum iOS version: to be confirmed (iOS 16 or 17 recommended).

- UI framework: SwiftUI.

- Map rendering: MapLibre.

- Storage: SQLite.

- Base map: to be confirmed. Offline strategy must be decided before build starts (see Open Decisions section).

- Internet requirement: none. The app must be fully functional offline once data and base maps are loaded.

# 3. Screen Layout

There is one main screen: the map. Everything else is a secondary screen reached from the More button.

## 3.1 Main map screen

The map fills the screen edge to edge. Overlaid on top:

- Bottom bar (always visible, four buttons): Pin · Record · Measure · More.

- North arrow (top corner, always visible): small, glanceable, also acts as a control. See section 7.

- Scale bar (bottom-left, always visible): metric units.

- Legend (top corner, opposite the north arrow): floating, collapsible, shows visible layers only.

- GPS accuracy indicator: small, near the position dot. Shows current accuracy in metres.

## 3.2 Secondary screens (reached via More)

- Maps — list of saved projects, import a .fieldmap, switch active map.

- Layers — list of layers in the current map, toggle visibility, toggle labels, edit style.

- Settings — base map source, app preferences, about.

Rules for all screens:

- No nested menus. Maximum two taps to reach any setting.

- Large touch targets (minimum 44pt).

- No styling controls anywhere on the map canvas.

# 4. Data Model

## 4.1 Project (also called a Map)

- id

- name

- createdAt

- formatVersion (for .fieldmap compatibility)

## 4.2 Layer

- id

- projectId

- name

- geometryType — point, line, or polygon

- style — colour, opacity, line width, point symbol

- displayField — the attribute used for labels

- isVisible

- labelsEnabled

## 4.3 Feature

- id

- layerId

- geometry — GeoJSON, WGS84

- properties — original attributes from the source file

## 4.4 Pin

- id

- projectId

- sequenceNumber — never reused within a project

- lat, lon (decimal degrees)

- altitude

- accuracy

- timestamp

- note (optional, free text)

## 4.5 Track

- id

- projectId

- name — auto-generated, sequential

- points — array of { lat, lon, altitude, timestamp }

- distance

- duration

- startTime

- endTime

# 5. Import

## 5.1 Supported formats

- Zipped shapefile (.zip containing .shp, .shx, .dbf, .prj)

- KML and KMZ

- GPX

- GeoJSON

- .fieldmap (the app's own project format)

## 5.2 Behaviour

- No setup screens. The user picks a file (via Files app, share sheet, or AirDrop) and the app processes it automatically.

- On success, the new layers appear and the map auto-zooms to fit them.

- On failure, a clear error message is shown. No partial imports.

## 5.3 Processing pipeline

- Detect file type from extension and contents.

- Validate structure.

- Parse geometry and attributes.

- Convert to GeoJSON internally.

- Reproject to WGS84 (EPSG:4326). Reject files that cannot be reprojected.

- Split mixed-geometry files into separate point, line, and polygon layers.

- Auto-pick display field: prefer an attribute called 'name' (case-insensitive); otherwise use the first text attribute.

- Discard source styling. Apply default style based on geometry type and a colour rotation.

- Simplify geometry if it exceeds [threshold to be set during testing — suggested starting point: 50,000 vertices per layer].

- Persist to SQLite.

- Render.

# 6. Export

## 6.1 .fieldmap (primary)

A ZIP archive that captures everything needed to reproduce the map exactly.

Contents:

- project.json — id, name, createdAt, formatVersion

- layers/*.geojson — one file per layer, WGS84

- styles.json — style and label settings per layer

- pins.geojson — all pins

- tracks/*.geojson — one file per track

- metadata.json — capture timestamps, app version, format version

Requirement:

- Importing a .fieldmap on a fresh install must reproduce the original map exactly — same layers, same styles, same pins, same tracks, same labels.

## 6.2 GeoJSON (secondary)

- Export individual layers, pins, or tracks as standalone GeoJSON files in WGS84.

- Used when the user wants to take their data into another tool.

## 6.3 Sharing

- Files are written via the iOS share sheet. The user picks the destination (Files app, AirDrop, email, third-party app).

# 7. Navigation and Orientation

## 7.1 North arrow

A small north arrow is visible in a top corner of the map at all times. It rotates to always point at true north.

Tap behaviour:

- If the map is rotated → tap snaps the map back to north-up.

- If the map is north-up → tap toggles auto-orientation (heading-up) on or off.

- When auto-orient is active, the arrow becomes solid/filled to make the state visible.

Compass behaviour uses standard iOS magnetometer heading. No special handling for magnetic interference required in v1.

## 7.2 Scale bar

- Bottom-left corner, always visible.

- Metric units only (metres / kilometres).

- Updates with zoom level.

## 7.3 Tap a feature

Tapping any feature on the map (imported feature, pin, or track point) shows a small info panel with:

- The feature's name (from its display field, or sequence number for pins)

- Distance from the user's current GPS position

- Bearing from the user's current GPS position (decimal degrees, true north)

- Coordinates (decimal degrees, six decimal places)

No routing. No turn-by-turn. No path-following.

# 8. Measure Tool

The measure tool is a primary, frequently-used feature. It earns its place in the bottom bar.

## 8.1 Behaviour

- Tap Measure → tool becomes active. The button shows an active state.

- Start point is automatically the user's current GPS location.

- End point is set by a single tap on the map.

- A line is drawn between the two points showing distance (metres or kilometres) and bearing (decimal degrees, true north).

- Tapping a different point updates the end point.

- Tapping Measure again, or any other bottom-bar button, exits the tool and clears the line.

## 8.2 Rules

- Single segment only. No multi-point or polyline measurement.

- Geodesic calculation (great-circle distance), not planar.

- Never saved. The measure line is purely temporary.

- If GPS is unavailable, the tool shows a clear message and is disabled until GPS returns.

# 9. Pins

## 9.1 Creating a pin

- Tap Pin button → drops a pin at the user's current GPS location.

- Long-press the map → drops a pin at that point on the map.

## 9.2 Numbering

- Pins are numbered sequentially within a project: 1, 2, 3, …

- Numbers are never reused. If pin 5 is deleted, the next new pin is 6, not 5.

## 9.3 Interacting with pins

- Tap a pin → shows info panel with coordinates, altitude, accuracy, timestamp, optional note, distance and bearing from current GPS.

- The info panel has an Edit Note button and a Delete button.

- Delete requires confirmation (a tap on Delete opens a confirm dialog with Delete and Cancel).

## 9.4 Undo

- After any pin creation or deletion, an Undo button appears briefly (5 seconds) at the top of the screen.

- Undo affects only the most recent pin action.

- Undo does not persist across app launches.

## 9.5 GPS unavailable

- If the user taps Pin and there is no current GPS fix, a message appears: 'No GPS signal. Try long-pressing the map to drop a pin manually.'

- The Pin button is greyed out (but not hidden) when GPS is unavailable.

# 10. Track Recording

## 10.1 Starting and stopping

- Tap Record → recording starts immediately. The button turns red. A small banner appears at the top: 'Recording — [duration] — [distance]'.

- Tap Record again → a Stop button slides up above the bottom bar. Tap Stop to end recording. (Two-tap stop prevents accidental termination.)

- On stop, the track is auto-saved with a default name: 'Track YYYY-MM-DD HH:MM'.

- The user can rename the track from the saved Tracks list later.

## 10.2 GPS sampling

- Distance-based sampling: capture a point every 3 metres of movement.

- Plus a time-based safety net: capture a point at minimum every 30 seconds even if stationary.

- These values to be tuned during beta testing.

## 10.3 Background recording

- Recording continues when the app is backgrounded or the screen is locked.

- Requires 'Always' location permission. The app must clearly explain why during the permission prompt.

- iOS will display a blue bar / status indicator while tracking — this is unavoidable and the user should be told it's expected.

## 10.4 Crash safety

- During recording, points are written to SQLite at least every 30 seconds.

- If the app crashes or is killed, on next launch the user is asked: 'A track recording was interrupted. Recover it?' with Recover and Discard options.

## 10.5 Visual

- Tracks render as a bright orange line, 4pt wide, with a 1pt dark grey outline for contrast.

- Tracks always render above all imported layers and below pins.

- Tracks can be hidden/shown from the Layers screen, like any other layer.

# 11. Layers and Styling

All styling lives in the Layers screen, reached via the More button. Styling controls never appear on the map canvas.

## 11.1 Layers screen

Shows a list of all layers in the current project. Each row shows:

- Layer name

- Geometry type icon (point, line, polygon)

- Visibility toggle

- Labels toggle

- A small swatch showing the layer's colour

Tapping a layer opens the layer detail screen.

## 11.2 Layer detail screen

- Rename the layer

- Choose colour (from a fixed palette of ~12 distinct colours)

- Adjust opacity (slider, 0–100%)

- Adjust line width (for lines and polygon outlines): 1pt, 2pt, 3pt, 5pt

- Choose point symbol (for point layers): circle, square, triangle, diamond, star, pin

- Toggle labels on/off

- Choose display field (defaults to auto-picked field; user can override)

- Delete the layer (with confirmation)

## 11.3 Style rules

- One style per layer. No per-feature styling.

- Polygons: filled with the chosen colour at the chosen opacity, outlined with the same colour at full opacity at the chosen line width.

- Labels are always black text with a white halo. The user cannot change label appearance.

- Labels declutter automatically (no overlapping text).

- Polygons are not labelled by default. Labels can be turned on but the developer should confirm visual quality before shipping.

# 12. Legend

- Floats in a top corner of the map (opposite the north arrow).

- Collapsible — tap to expand or collapse.

- Shows only currently visible layers.

- Each row: colour swatch, point symbol if applicable, layer name.

- Read-only. No editing from the legend. (Tapping a layer name in the legend does NOT take you to the Layers screen — that would create two paths to the same place and add complexity.)

# 13. Metadata

Automatically captured for every pin and every track point. No manual forms.

- Latitude, longitude (decimal degrees, six decimal places)

- Altitude (metres)

- GPS accuracy (metres)

- Timestamp (ISO 8601, UTC)

Pins also have an optional free-text note, editable from the pin info panel.

# 14. Interaction Safety

- Destructive actions (delete pin, delete layer, delete track) require a confirmation dialog.

- Track recording requires two taps to stop.

- Map taps are ignored if the user is mid-pan or mid-pinch.

- The active tool (Pin, Record, Measure) is always visually highlighted in the bottom bar so the user knows what mode they're in.

- Long-press to place a pin requires a deliberate hold (~0.5 seconds) to avoid accidental drops.

# 15. Performance

- Geometry is simplified on import if it exceeds the vertex threshold.

- Label rendering is limited per zoom level to keep the map clean and the frame rate smooth.

- The app prioritises smooth pan and zoom over rendering completeness.

- Assume data is pre-processed externally where possible. The app is not a substitute for a desktop GIS.

# 16. Scope Boundaries — Not in v1

These are deliberate exclusions. Building any of them in v1 will compromise the speed-and-clarity goal.

- Geometry editing (moving vertices, reshaping features)

- Attribute editing or attribute tables

- Coordinate system selection (always WGS84)

- Multiple coordinate display formats (decimal degrees only)

- Per-feature styling

- Routing, directions, or path following

- Voice guidance

- Accounts, login, sync, cloud features

- Photo or video attachments to pins

- Custom base map import (mbtiles, etc.) — possible v2

- Multi-track or multi-pin selection and bulk operations

- Search within data

# 17. Open Decisions

These must be resolved before development starts.

- Base map source and offline strategy. Options include: bundled OpenStreetMap tiles, downloadable regions via the app, user-supplied .mbtiles files, or a paid provider with offline caching.

- Minimum iOS version (iOS 16, 17, or 18).

- Maximum imported file size before the app warns or rejects.

- Geometry simplification threshold (vertices per layer).

Decisions to confirm but flagged with current best-guess defaults:

- GPS sampling: every 3m of movement, plus every 30s as a safety net.

- Track autosave interval: every 30 seconds.

- Undo window: 5 seconds after the action.

- Long-press duration to drop a manual pin: 0.5 seconds.

- Colour palette: 12 distinct, high-contrast colours suitable for outdoor viewing.

# 18. Build Phases

Suggested order. Each phase produces a working, testable app.

## Phase 0 — Setup (1 week)

- Xcode project, MapLibre integration, SQLite schema, base map decision implemented, GPS dot on map.

## Phase 1 — Map and import (3–4 weeks)

- Importer for GeoJSON, then KML/KMZ, then GPX, then zipped shapefile.

- Layers screen with visibility toggle, label toggle, and full styling controls.

- Legend on map canvas.

- Auto-zoom to extent on import.

- Tap-feature info panel.

## Phase 2 — Capture (3 weeks)

- Pins: create, delete, undo, sequence numbering, info panel, optional notes.

- Tracks: record with tap-to-start and two-tap stop, GPS sampling, background recording, crash recovery, autosave.

- Project (Map) management screen — create, switch, delete.

## Phase 3 — Tools and orientation (2 weeks)

- Measure tool with GPS-to-tap distance and bearing.

- North arrow with tap-to-snap and tap-to-toggle auto-orient.

- Scale bar.

- GPS accuracy indicator.

## Phase 4 — Export and polish (2–3 weeks)

- .fieldmap export and import (round-trip tested on a fresh install).

- GeoJSON export per layer, per pin, per track.

- Edge cases: no GPS, full storage, very large files, app killed mid-recording, app backgrounded mid-recording.

- Visual polish.

## Phase 5 — Beta (2 weeks minimum, in real field conditions)

- Hand to real users (cavers, gorge-walkers, surveyors). Fix what breaks.

Total estimate: 11–14 weeks of solo development plus beta.

# 19. Summary

This is a calm, opinionated, single-purpose app. It does fewer things than Map Plus on purpose. Its competitive advantage is not breadth of features — it is the speed and confidence with which a user can drop a pin, record a track, or measure a distance while standing in a wet cave at 3am.

If at any point during development a feature is being added that does not directly serve that user, that feature should be cut or deferred.